Design and Verification of a RISC-V VeeR EL2 Based Smart Peripheral Control SoC

Interface Specifications, Memory Map and Bus Protocol Details

Figure 1: Proposed VeeR EL2 Based Smart Peripheral Control SoC Architecture

# 1) Proposed Architecture :

The proposed SoC is built around a RISC-V VeeR EL2 RV32IMC processor core. The core contains the IFU, LSU, integer pipeline, CSR, interrupt interface, debug interface, DMA slave port and Fence/PMP support. The LSU is configured as the AXI4-Lite master interface for system-level data transactions. ICCM and DCCM are connected directly to the processor through the instruction and data closely-coupled memory interfaces.

| Block | Interface / Signals | Function |
| --- | --- | --- |
| VeeR EL2 RISC-V Core | IFU, LSU, CSR, Interrupt Interface, Debug, DMA Slave Port | RV32IMC processor and system-bus master. |
| ICCM | Instr. Bus (direct) | Instruction closely-coupled memory for instruction fetch. |
| DCCM | Data Bus (direct) | Data closely-coupled memory for core-local data storage. |
| AXI4-Lite System Bus Interconnect | AXI4-Lite master/slave channels | Address decode and routing between the LSU and peripherals. |
| PIC | Interrupt inputs + interrupt-to-core | Prioritizes and routes peripheral interrupts to the VeeR EL2 core. |
| UART | AXI4-Lite + UART TX/RX | Host-computer serial communication. |
| Timer | AXI4-Lite + timer controls | Programmable counting/timing and interrupt generation. |
| GPIO | AXI4-Lite + GPIO I/O | General-purpose digital input/output and interrupt-on-change. |
| SPI Controller | AXI4-Lite + MOSI/MISO/SCLK/SS | Serial communication with an SPI peripheral. |
| I²C Controller | AXI4-Lite + SDA/SCL | Two-wire serial communication with an I²C device. |
| PWM Generator | AXI4-Lite + PWM Out | Programmable duty-cycle/period waveform generation. |
| Clock & Reset Unit | clk_i, rst_n | Provides system clock/reset handling. |
| System Control & Configuration | Configuration/status controls | Reset, clock-gating and SoC configuration support shown in the architecture. |

# 2) Memory Map :

The address map below follows the address regions shown in the proposed architecture.

| Address Range / Base | Block | Access / Purpose |
| --- | --- | --- |
| 0x0000_0000 | ICCM | Instruction closely-coupled memory; accessed directly by the IFU. |
| 0x1000_0000 | DCCM | Data closely-coupled memory; accessed directly through the core data interface. |
| 0x2000_0000 | UART | Memory-mapped UART registers. |
| 0x2000_1000 | Timer | Memory-mapped timer registers. |
| 0x2000_2000 | GPIO | Memory-mapped GPIO registers. |
| 0x2000_3000 | SPI | Memory-mapped SPI controller registers. |
| 0x2000_4000 | I²C | Memory-mapped I²C controller registers. |
| 0x2000_5000 | PWM | Memory-mapped PWM generator registers. |
| Other regions | Reserved / other VeeR EL2 regions | Available for additional SoC functions and future expansion. |

Note: The supplied architecture identifies ICCM at 0x0000_0000, DCCM at 0x1000_0000, and the peripheral region beginning at 0x2000_0000. Exact memory sizes and register offsets should be finalized when the RTL/register specification is frozen.

# 3) Interface Specifications :

## 3.1 VeeR EL2 LSU / AXI4-Lite Interface

| Signal | Dir. at Core | Width | Description |
| --- | --- | --- | --- |
| lsu_awaddr | Output | 32 | Write-address channel address for a peripheral transaction. |
| lsu_awvalid | Output | 1 | Indicates a valid write address. |
| lsu_awready | Input | 1 | Interconnect acceptance of the write address. |
| lsu_wdata | Output | 32 | Write data from the LSU. |
| lsu_wstrb | Output | 4 | Byte write strobes for the 32-bit write data. |
| lsu_wvalid | Output | 1 | Indicates valid write data. |
| lsu_wready | Input | 1 | Interconnect acceptance of write data. |
| lsu_bresp | Input | 2 | AXI4-Lite write response returned by the interconnect. |
| lsu_bvalid | Input | 1 | Indicates a valid write response. |
| lsu_bready | Output | 1 | Indicates the LSU is ready to accept the write response. |
| lsu_araddr | Output | 32 | Read-address channel address for a peripheral transaction. |
| lsu_arvalid | Output | 1 | Indicates a valid read address. |
| lsu_arready | Input | 1 | Interconnect acceptance of the read address. |
| lsu_rdata | Input | 32 | Read data returned by the selected AXI4-Lite peripheral. |
| lsu_rresp | Input | 2 | AXI4-Lite read response. |
| lsu_rvalid | Input | 1 | Indicates valid read data and response. |
| lsu_rready | Output | 1 | Indicates the LSU is ready to accept read data. |

## 3.2 ICCM / DCCM Interfaces

| Signal | Direction | Description |
| --- | --- | --- |
| iccm_addr | Core → ICCM | Instruction address for direct IFU access. |
| iccm_en | Core → ICCM | Instruction-memory access enable. |
| iccm_rdata | ICCM → Core | Instruction data returned to the IFU. |
| dccm_addr | Core → DCCM | Data address for direct LSU/DCCM access. |
| dccm_wdata | Core → DCCM | Data written to DCCM. |
| dccm_rdata | DCCM → Core | Data read from DCCM. |
| dccm_en | Core → DCCM | Data-memory access enable. |
| dccm_we | Core → DCCM | Data write enable/control. |

## 3.3 UART (Base 0x2000_0000)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| uart_awaddr | Input | 32 | AXI4-Lite write address. |
| uart_awvalid | Input | 1 | Valid write address. |
| uart_awready | Output | 1 | UART accepts the write address. |
| uart_wdata | Input | 32 | Write data. |
| uart_wstrb | Input | 4 | Byte write strobes. |
| uart_wvalid | Input | 1 | Valid write data. |
| uart_wready | Output | 1 | UART accepts write data. |
| uart_bresp | Output | 2 | Write response. |
| uart_bvalid | Output | 1 | Valid write response. |
| uart_bready | Input | 1 | Interconnect accepts write response. |
| uart_araddr | Input | 32 | AXI4-Lite read address. |
| uart_arvalid | Input | 1 | Valid read address. |
| uart_arready | Output | 1 | UART accepts the read address. |
| uart_rdata | Output | 32 | Read data. |
| uart_rresp | Output | 2 | Read response. |
| uart_rvalid | Output | 1 | Valid read data/response. |
| uart_rready | Input | 1 | Interconnect accepts read data. |
| uart_tx | Output | 1 | Serial transmit line to host computer. |
| uart_rx | Input | 1 | Serial receive line from host computer. |
| uart_irq | Output | 1 | UART interrupt indication to PIC, if enabled by the implementation. |

Suggested register-level signals/data: TX_DATA, RX_DATA, STATUS, CTRL, TX_READY, RX_VALID and FRAME_ERR.

## 3.4 Timer (Base 0x2000_1000)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| timer_awaddr | Input | 32 | AXI4-Lite write address. |
| timer_awvalid | Input | 1 | Valid write address. |
| timer_awready | Output | 1 | Timer accepts the write address. |
| timer_wdata | Input | 32 | Write data. |
| timer_wstrb | Input | 4 | Byte write strobes. |
| timer_wvalid | Input | 1 | Valid write data. |
| timer_wready | Output | 1 | Timer accepts write data. |
| timer_bresp | Output | 2 | Write response. |
| timer_bvalid | Output | 1 | Valid write response. |
| timer_bready | Input | 1 | Interconnect accepts write response. |
| timer_araddr | Input | 32 | AXI4-Lite read address. |
| timer_arvalid | Input | 1 | Valid read address. |
| timer_arready | Output | 1 | Timer accepts the read address. |
| timer_rdata | Output | 32 | Read data. |
| timer_rresp | Output | 2 | Read response. |
| timer_rvalid | Output | 1 | Valid read data/response. |
| timer_rready | Input | 1 | Interconnect accepts read data. |
| timer_count | Internal/Register | Implementation-defined | Current programmable counter value. |
| timer_compare | Internal/Register | Implementation-defined | Compare/match value. |
| timer_enable | Internal/Register | 1 | Timer enable. |
| timer_expire | Internal | 1 | Indicates timer expiry/match. |
| timer_irq | Output | 1 | Timer interrupt to PIC. |

Suggested register-level controls: COUNT, COMPARE, CTRL and STATUS.

## 3.5 GPIO (Base 0x2000_2000)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| gpio_awaddr | Input | 32 | AXI4-Lite write address. |
| gpio_awvalid | Input | 1 | Valid write address. |
| gpio_awready | Output | 1 | GPIO accepts the write address. |
| gpio_wdata | Input | 32 | Write data. |
| gpio_wstrb | Input | 4 | Byte write strobes. |
| gpio_wvalid | Input | 1 | Valid write data. |
| gpio_wready | Output | 1 | GPIO accepts write data. |
| gpio_bresp | Output | 2 | Write response. |
| gpio_bvalid | Output | 1 | Valid write response. |
| gpio_bready | Input | 1 | Interconnect accepts write response. |
| gpio_araddr | Input | 32 | AXI4-Lite read address. |
| gpio_arvalid | Input | 1 | Valid read address. |
| gpio_arready | Output | 1 | GPIO accepts the read address. |
| gpio_rdata | Output | 32 | Read data. |
| gpio_rresp | Output | 2 | Read response. |
| gpio_rvalid | Output | 1 | Valid read data/response. |
| gpio_rready | Input | 1 | Interconnect accepts read data. |
| gpio_in | Input | N | External digital input pins. |
| gpio_out | Output | N | External digital output pins. |
| gpio_oe | Output | N | Optional output-enable for bidirectional implementation. |
| gpio_irq | Output | 1 | Interrupt-on-change indication to PIC. |

Suggested register-level controls: GPIO_DATA_OUT, GPIO_DATA_IN, GPIO_DIR and interrupt-on-change status/control.

## 3.6 SPI Controller (Base 0x2000_3000)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| spi_awaddr | Input | 32 | AXI4-Lite write address. |
| spi_awvalid | Input | 1 | Valid write address. |
| spi_awready | Output | 1 | SPI accepts the write address. |
| spi_wdata | Input | 32 | Write data. |
| spi_wstrb | Input | 4 | Byte write strobes. |
| spi_wvalid | Input | 1 | Valid write data. |
| spi_wready | Output | 1 | SPI accepts write data. |
| spi_bresp | Output | 2 | Write response. |
| spi_bvalid | Output | 1 | Valid write response. |
| spi_bready | Input | 1 | Interconnect accepts write response. |
| spi_araddr | Input | 32 | AXI4-Lite read address. |
| spi_arvalid | Input | 1 | Valid read address. |
| spi_arready | Output | 1 | SPI accepts the read address. |
| spi_rdata | Output | 32 | Read data. |
| spi_rresp | Output | 2 | Read response. |
| spi_rvalid | Output | 1 | Valid read data/response. |
| spi_rready | Input | 1 | Interconnect accepts read data. |
| spi_mosi | Output | 1 | Master-out serial data. |
| spi_miso | Input | 1 | Master-in serial data. |
| spi_sclk | Output | 1 | SPI serial clock. |
| spi_cs_n | Output | 1 or N | Slave/chip-select signal. |
| spi_tx_data | Internal/Register | 8 or implementation-defined | Transmit data. |
| spi_rx_data | Internal/Register | 8 or implementation-defined | Received data. |
| spi_start | Internal/Register | 1 | Transfer start control. |
| spi_busy | Internal | 1 | Transfer active status. |
| spi_done | Internal | 1 | Transfer completion status. |
| spi_irq | Output | 1 | SPI interrupt to PIC. |

## 3.7 I²C Controller (Base 0x2000_4000)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| i2c_awaddr | Input | 32 | AXI4-Lite write address. |
| i2c_awvalid | Input | 1 | Valid write address. |
| i2c_awready | Output | 1 | I²C accepts the write address. |
| i2c_wdata | Input | 32 | Write data. |
| i2c_wstrb | Input | 4 | Byte write strobes. |
| i2c_wvalid | Input | 1 | Valid write data. |
| i2c_wready | Output | 1 | I²C accepts write data. |
| i2c_bresp | Output | 2 | Write response. |
| i2c_bvalid | Output | 1 | Valid write response. |
| i2c_bready | Input | 1 | Interconnect accepts write response. |
| i2c_araddr | Input | 32 | AXI4-Lite read address. |
| i2c_arvalid | Input | 1 | Valid read address. |
| i2c_arready | Output | 1 | I²C accepts the read address. |
| i2c_rdata | Output | 32 | Read data. |
| i2c_rresp | Output | 2 | Read response. |
| i2c_rvalid | Output | 1 | Valid read data/response. |
| i2c_rready | Input | 1 | Interconnect accepts read data. |
| i2c_sda | Bidirectional | 1 | I²C serial data line. |
| i2c_scl | Bidirectional | 1 | I²C serial clock line. |
| i2c_tx_data | Internal/Register | 8 or implementation-defined | Transmit data. |
| i2c_rx_data | Internal/Register | 8 or implementation-defined | Received data. |
| i2c_addr | Internal/Register | 7/10 or implementation-defined | Target device address. |
| i2c_start | Internal/Register | 1 | Start-condition control. |
| i2c_stop | Internal/Register | 1 | Stop-condition control. |
| i2c_ack | Internal | 1 | Acknowledgement status/control. |
| i2c_busy | Internal | 1 | Transaction active. |
| i2c_done | Internal | 1 | Transaction completion. |
| i2c_error | Internal | 1 | Error status. |
| i2c_irq | Output | 1 | I²C interrupt to PIC. |

## 3.8 PWM Generator (Base 0x2000_5000)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| pwm_awaddr | Input | 32 | AXI4-Lite write address. |
| pwm_awvalid | Input | 1 | Valid write address. |
| pwm_awready | Output | 1 | PWM accepts the write address. |
| pwm_wdata | Input | 32 | Write data. |
| pwm_wstrb | Input | 4 | Byte write strobes. |
| pwm_wvalid | Input | 1 | Valid write data. |
| pwm_wready | Output | 1 | PWM accepts write data. |
| pwm_bresp | Output | 2 | Write response. |
| pwm_bvalid | Output | 1 | Valid write response. |
| pwm_bready | Input | 1 | Interconnect accepts write response. |
| pwm_araddr | Input | 32 | AXI4-Lite read address. |
| pwm_arvalid | Input | 1 | Valid read address. |
| pwm_arready | Output | 1 | PWM accepts the read address. |
| pwm_rdata | Output | 32 | Read data. |
| pwm_rresp | Output | 2 | Read response. |
| pwm_rvalid | Output | 1 | Valid read data/response. |
| pwm_rready | Input | 1 | Interconnect accepts read data. |
| pwm_enable | Internal/Register | 1 | PWM enable. |
| pwm_period | Internal/Register | Implementation-defined | PWM period value. |
| pwm_duty | Internal/Register | Implementation-defined | Duty-cycle value. |
| pwm_counter | Internal | Implementation-defined | Timing counter. |
| pwm_compare | Internal | Implementation-defined | Comparator threshold. |
| pwm_out | Output | 1 | Generated PWM waveform to external load. |

## 3.9 Programmable Interrupt Controller (PIC)

| Signal | Direction | Description |
| --- | --- | --- |
| irq_uart | Peripheral → PIC | UART interrupt source. |
| irq_timer | Peripheral → PIC | Timer expiry/match interrupt source. |
| irq_gpio | Peripheral → PIC | GPIO interrupt-on-change source. |
| irq_spi | Peripheral → PIC | SPI transfer-completion interrupt source. |
| irq_i2c | Peripheral → PIC | I²C completion/error interrupt source. |
| irq_sources | Input vector | Aggregated peripheral interrupt sources; PIC architecture shown supports up to 255 sources. |
| irq_to_core | PIC → Core | Prioritized interrupt output to the VeeR EL2 interrupt interface. |
| irq_enable | Internal/Register | Per-source interrupt enable control. |
| irq_pending | Internal/Register | Pending interrupt state. |
| irq_priority | Internal/Register | Interrupt priority information. |

# 4) Bus Protocol Details :

The proposed architecture uses AXI4-Lite as the system bus connected to the VeeR EL2 LSU. The interconnect performs address decoding and routes LSU transactions to the selected peripheral slave. ICCM and DCCM remain direct core-local interfaces and are not treated as ordinary peripheral-bus slaves.

| Connection | Protocol | Role | Description |
| --- | --- | --- | --- |
| VeeR EL2 LSU → AXI4-Lite Interconnect | AXI4-Lite | Master | Core load/store transactions and memory-mapped peripheral accesses. |
| AXI4-Lite Interconnect → UART | AXI4-Lite | Slave | UART register read/write access. |
| AXI4-Lite Interconnect → Timer | AXI4-Lite | Slave | Timer register access. |
| AXI4-Lite Interconnect → GPIO | AXI4-Lite | Slave | GPIO register access. |
| AXI4-Lite Interconnect → SPI | AXI4-Lite | Slave | SPI control/data register access. |
| AXI4-Lite Interconnect → I²C | AXI4-Lite | Slave | I²C control/data register access. |
| AXI4-Lite Interconnect → PWM | AXI4-Lite | Slave | PWM configuration register access. |
| Core IFU ↔ ICCM | Direct core-local interface | Direct | Instruction fetch path. |
| Core LSU ↔ DCCM | Direct core-local interface | Direct | Data access path. |
| PIC → VeeR EL2 | Interrupt interface | Interrupt | Prioritized external/platform interrupt delivery. |

## 4.1 AXI4-Lite Transaction Signals
AXI4-Lite is used for the processor-to-peripheral control path. It is a non-burst interface with independent write-address, write-data, write-response, read-address and read-data channels.

| Signal | Meaning |
| --- | --- |
| AWADDR | Write-address channel address. |
| AWVALID/AWREADY | Write-address valid/ready handshake. |
| WDATA/WSTRB | Write-data channel and byte strobes. |
| WVALID/WREADY | Write-data valid/ready handshake. |
| BRESP/BVALID/BREADY | Write response and response handshake. |
| ARADDR | Read-address channel address. |
| ARVALID/ARREADY | Read-address valid/ready handshake. |
| RDATA/RRESP | Read-data and read-response channel. |
| RVALID/RREADY | Read-data valid/ready handshake. |
| AXI4-Lite channels | AXI4-Lite uses independent read and write address/data/response channels; no bursts are used. |

## 4.2 Address Decode and Routing

The interconnect compares the LSU AWADDR/ARADDR against the peripheral address ranges.

Only the selected peripheral receives an active slave transaction.

The selected peripheral returns RDATA, READY and BRESP/RRESP toward the interconnect.

The interconnect multiplexes the selected slave RDATA back to the LSU.

Peripheral registers are accessed through memory-mapped read/write transactions.

ICCM and DCCM are handled through their direct core-local interfaces rather than through the peripheral AXI4-Lite path.

## 4.3 Interrupt Routing

Peripheral interrupt outputs are connected to the PIC. The PIC collects and prioritizes the sources and presents the resulting interrupt request to the VeeR EL2 interrupt interface.

| Peripheral | Interrupt Signal | PIC Destination |
| --- | --- | --- |
| UART | irq_uart | PIC interrupt source |
| Timer | irq_timer | PIC interrupt source |
| GPIO | irq_gpio | PIC interrupt source |
| SPI | irq_spi | PIC interrupt source |
| I²C | irq_i2c | PIC interrupt source |
| PIC | irq_to_core | VeeR EL2 interrupt interface |

## 4.4 External I/O Signals

| External Device / Interface | Signals |
| --- | --- |
| Host Computer / Serial Terminal | uart_tx, uart_rx |
| External Digital I/O | gpio_in, gpio_out, optional gpio_oe |
| SPI Peripheral | spi_mosi, spi_miso, spi_sclk, spi_cs_n |
| I²C Device | i2c_sda, i2c_scl |
| External Load | pwm_out |
| System Clock / Reset | clk_i, rst_n |

## 4.5 Verification Signals

For RTL verification, the following signals should be made observable in the testbench or through an ILA/debug interface where required:

| Verification Area | Primary Signals |
| --- | --- |
| Reset / Clock | clk_i, rst_n |
| AXI4-Lite | HADDR, HWRITE, HTRANS, HSIZE, HWDATA, HRDATA, HREADY, HRESP |
| Interrupts | irq_uart, irq_timer, irq_gpio, irq_spi, irq_i2c, irq_to_core |
| UART | uart_tx, uart_rx |
| SPI | spi_sclk, spi_cs_n, spi_mosi, spi_miso, spi_done, spi_irq |
| I²C | i2c_scl, i2c_sda, i2c_done, i2c_error, i2c_irq |
| Timer | timer_count, timer_compare, timer_expire, timer_irq |
| GPIO | gpio_in, gpio_out, gpio_oe, gpio_irq |
| PWM | pwm_enable, pwm_period, pwm_duty, pwm_out |

Implementation note: The block diagram is the architectural reference for this document. Signal names marked as proposed/internal are intended to be mapped to the exact VeeR EL2 RTL and peripheral RTL port names during implementation. The architecture uses AXI4-Lite for the LSU system bus, as shown in the supplied block diagram.