Signal List — RISC-V VeeR EL2 Based Smart Peripheral Control SoC (AXI4-Lite Peripheral Path)

Direction is given relative to the block itself (Input = signal received by the block, Output = signal driven by the block), unless noted as "Top-Level" where direction is relative to the SoC package pins.

---

# 1. Top-Level SoC External I/O

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | Input | 1 | System clock |
| rst_n | Input | 1 | Active-low asynchronous system reset |
| uart_tx | Output | 1 | UART transmit line to host computer |
| uart_rx | Input | 1 | UART receive line from host computer |
| spi_sclk | Output | 1 | SPI serial clock to external SPI device |
| spi_mosi | Output | 1 | SPI master-out-slave-in data |
| spi_miso | Input | 1 | SPI master-in-slave-out data |
| spi_cs_n | Output | 1..N | SPI chip-select (one per external device) |
| i2c_scl | Inout | 1 | I²C serial clock (open-drain) |
| i2c_sda | Inout | 1 | I²C serial data (open-drain) |
| pwm_out | Output | 1 | PWM waveform output to actuator/LED/motor driver |
| gpio_pins | Inout | N (config.) | General-purpose digital I/O bus |
| jtag_tck/tms/tdi/tdo/trst_n | Input/Output | 1 each | Optional debug port, if VeeR debug module is brought out to top level |

---

# 2. VeeR EL2 Core — LSU AXI4-Lite Master Interface

These are the core's system-bus AXI4-Lite master signals for the peripheral-access path. Direction is from the core's perspective. The interface is split into the AXI write-address, write-data, write-response, read-address, and read-data channels.

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| lsu_awaddr | Output | 32 | Write address for the current LSU transaction |
| lsu_awvalid | Output | 1 | Write-address channel valid |
| lsu_awready | Input | 1 | Write-address channel ready from the interconnect |
| lsu_wdata | Output | 64 | Write data (core LSU data width up to 64-bit) |
| lsu_wstrb | Output | 8 | Write byte strobes; one strobe per byte of the 64-bit write data |
| lsu_wvalid | Output | 1 | Write-data channel valid |
| lsu_wready | Input | 1 | Write-data channel ready from the interconnect |
| lsu_bresp | Input | 2 | Write response returned by the interconnect |
| lsu_bvalid | Input | 1 | Write response valid |
| lsu_bready | Output | 1 | Core ready to accept the write response |
| lsu_araddr | Output | 32 | Read address for the current LSU transaction |
| lsu_arvalid | Output | 1 | Read-address channel valid |
| lsu_arready | Input | 1 | Read-address channel ready from the interconnect |
| lsu_rdata | Input | 64 | Read data returned from the addressed slave |
| lsu_rresp | Input | 2 | Read response returned by the interconnect |
| lsu_rvalid | Input | 1 | Read-data channel valid |
| lsu_rready | Output | 1 | Core ready to accept read data |
| lsu_clk_en | Output | 1 | LSU clock-enable/gating control (implementation-dependent) |

Analogous IFU, Debug, and DMA-slave AHB-Lite/AXI4 interfaces may exist on the core, but the peripheral-access path in this project is specified as AXI4-Lite and only the LSU-facing path is enumerated here.

---

# 3. AXI4-Lite Bus Interconnect

## 3a. Core-facing side (AXI4-Lite slave-side signals as seen by the LSU master)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| s_awaddr | Input | 32 | Write address received from the LSU AXI4-Lite master |
| s_awvalid | Input | 1 | Write-address channel valid from the core |
| s_awready | Output | 1 | Write-address channel ready to the core |
| s_wdata | Input | 64 | Write data received from the LSU AXI4-Lite master |
| s_wstrb | Input | 8 | Write byte strobes from the LSU AXI4-Lite master |
| s_wvalid | Input | 1 | Write-data channel valid from the core |
| s_wready | Output | 1 | Write-data channel ready to the core |
| s_bresp | Output | 2 | Write response returned to the LSU |
| s_bvalid | Output | 1 | Write response valid to the core |
| s_bready | Input | 1 | Core ready to accept write response |
| s_araddr | Input | 32 | Read address received from the LSU AXI4-Lite master |
| s_arvalid | Input | 1 | Read-address channel valid from the core |
| s_arready | Output | 1 | Read-address channel ready to the core |
| s_rdata | Output | 64 | Read data returned to the LSU |
| s_rresp | Output | 2 | Read response returned to the LSU |
| s_rvalid | Output | 1 | Read-data channel valid to the core |
| s_rready | Input | 1 | Core ready to accept read data |

## 3b. Per-peripheral AXI4-Lite slave-facing side (repeated for UART, Timer, GPIO, SPI, I²C, PWM)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_awaddr_<periph> | Output | 32 (or reg-offset width) | Decoded write address driven to the selected peripheral |
| m_awvalid_<periph> | Output | 1 | Write-address channel valid to the selected peripheral |
| m_awready_<periph> | Input | 1 | Write-address channel ready from the selected peripheral |
| m_wdata_<periph> | Output | 32 | Write data forwarded to the selected peripheral |
| m_wstrb_<periph> | Output | 4 | Write byte strobes for the 32-bit peripheral data path |
| m_wvalid_<periph> | Output | 1 | Write-data channel valid to the selected peripheral |
| m_wready_<periph> | Input | 1 | Write-data channel ready from the selected peripheral |
| m_bresp_<periph> | Input | 2 | Write response returned from the selected peripheral |
| m_bvalid_<periph> | Input | 1 | Write response valid from the selected peripheral |
| m_bready_<periph> | Output | 1 | Interconnect ready to accept the write response |
| m_araddr_<periph> | Output | 32 (or reg-offset width) | Decoded read address driven to the selected peripheral |
| m_arvalid_<periph> | Output | 1 | Read-address channel valid to the selected peripheral |
| m_arready_<periph> | Input | 1 | Read-address channel ready from the selected peripheral |
| m_rdata_<periph> | Input | 32 | Read data returned from the selected peripheral |
| m_rresp_<periph> | Input | 2 | Read response returned from the selected peripheral |
| m_rvalid_<periph> | Input | 1 | Read-data channel valid from the selected peripheral |
| m_rready_<periph> | Output | 1 | Interconnect ready to accept read data |

---

# 4. UART Controller

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| s_axi_uart_awaddr, s_axi_uart_awvalid, s_axi_uart_awready, s_axi_uart_wdata, s_axi_uart_wstrb, s_axi_uart_wvalid, s_axi_uart_wready | Input | — | AXI4-Lite slave write-address and write-data channel signals |
| s_axi_uart_bresp, s_axi_uart_bvalid, s_axi_uart_bready, s_axi_uart_araddr, s_axi_uart_arvalid, s_axi_uart_arready, s_axi_uart_rdata, s_axi_uart_rresp, s_axi_uart_rvalid, s_axi_uart_rready | Output | — | AXI4-Lite slave write-response, read-address and read-data channel signals |
| tx_data_reg (mem-mapped) | — | 8 | Register: byte to transmit |
| rx_data_reg (mem-mapped) | — | 8 | Register: last byte received |
| baud_div_reg (mem-mapped) | — | 16 | Register: baud-rate divisor |
| status_reg (mem-mapped) | — | — | Register: TX-busy, RX-valid, framing/overrun error flags |
| tx_serial | Output | 1 | Serial transmit line (drives uart_tx) |
| rx_serial | Input | 1 | Serial receive line (from uart_rx) |
| uart_irq | Output | 1 | Interrupt request to PIC (TX-empty / RX-ready) |

---

# 5. Timer

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| s_axi_tmr_awaddr, s_axi_tmr_awvalid, s_axi_tmr_awready, s_axi_tmr_wdata, s_axi_tmr_wstrb, s_axi_tmr_wvalid, s_axi_tmr_wready | Input | — | AXI4-Lite slave write-address and write-data channel signals |
| s_axi_tmr_bresp, s_axi_tmr_bvalid, s_axi_tmr_bready, s_axi_tmr_araddr, s_axi_tmr_arvalid, s_axi_tmr_arready, s_axi_tmr_rdata, s_axi_tmr_rresp, s_axi_tmr_rvalid, s_axi_tmr_rready | Output | — | AXI4-Lite slave write-response, read-address and read-data channel signals |
| load_reg (mem-mapped) | — | 32 | Register: count/reload value |
| count_reg (mem-mapped) | — | 32 | Register: current count (read-only) |
| ctrl_reg (mem-mapped) | — | — | Register: enable, mode (one-shot/periodic), prescaler |
| timer_irq | Output | 1 | Interrupt request to PIC on expiry |

---

# 6. GPIO

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| s_axi_gpio_awaddr, s_axi_gpio_awvalid, s_axi_gpio_awready, s_axi_gpio_wdata, s_axi_gpio_wstrb, s_axi_gpio_wvalid, s_axi_gpio_wready | Input | — | AXI4-Lite slave write-address and write-data channel signals |
| s_axi_gpio_bresp, s_axi_gpio_bvalid, s_axi_gpio_bready, s_axi_gpio_araddr, s_axi_gpio_arvalid, s_axi_gpio_arready, s_axi_gpio_rdata, s_axi_gpio_rresp, s_axi_gpio_rvalid, s_axi_gpio_rready | Output | — | AXI4-Lite slave write-response, read-address and read-data channel signals |
| dir_reg (mem-mapped) | — | N | Register: per-pin direction (0 = input, 1 = output) |
| out_reg (mem-mapped) | — | N | Register: output drive values |
| in_reg (mem-mapped) | — | N | Register: current input pin values (read-only) |
| int_en_reg (mem-mapped) | — | N | Register: per-pin interrupt-on-change enable |
| gpio_in | Input | N | Physical pin values sampled from gpio_pins |
| gpio_out | Output | N | Physical pin drive values to gpio_pins |
| gpio_oe | Output | N | Output-enable per pin (for tristate control at the pad) |
| gpio_irq | Output | 1 | Interrupt request to PIC on enabled pin change |

---

# 7. SPI Controller

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| s_axi_spi_awaddr, s_axi_spi_awvalid, s_axi_spi_awready, s_axi_spi_wdata, s_axi_spi_wstrb, s_axi_spi_wvalid, s_axi_spi_wready | Input | — | AXI4-Lite slave write-address and write-data channel signals |
| s_axi_spi_bresp, s_axi_spi_bvalid, s_axi_spi_bready, s_axi_spi_araddr, s_axi_spi_arvalid, s_axi_spi_arready, s_axi_spi_rdata, s_axi_spi_rresp, s_axi_spi_rvalid, s_axi_spi_rready | Output | — | AXI4-Lite slave write-response, read-address and read-data channel signals |
| tx_data_reg (mem-mapped) | — | 8/16/32 | Register: data to shift out |
| rx_data_reg (mem-mapped) | — | 8/16/32 | Register: data shifted in |
| ctrl_reg (mem-mapped) | — | — | Register: clock polarity/phase (CPOL/CPHA), clock divider, enable |
| status_reg (mem-mapped) | — | — | Register: transfer-done, TX/RX FIFO status flags |
| sclk_out | Output | 1 | Serial clock (drives spi_sclk) |
| mosi_out | Output | 1 | Master-out data (drives spi_mosi) |
| miso_in | Input | 1 | Master-in data (from spi_miso) |
| cs_n_out | Output | 1..N | Chip-select (drives spi_cs_n) |
| spi_irq | Output | 1 | Interrupt request to PIC on transfer completion |

---

# 8. I²C Controller

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| s_axi_i2c_awaddr, s_axi_i2c_awvalid, s_axi_i2c_awready, s_axi_i2c_wdata, s_axi_i2c_wstrb, s_axi_i2c_wvalid, s_axi_i2c_wready | Input | — | AXI4-Lite slave write-address and write-data channel signals |
| s_axi_i2c_bresp, s_axi_i2c_bvalid, s_axi_i2c_bready, s_axi_i2c_araddr, s_axi_i2c_arvalid, s_axi_i2c_arready, s_axi_i2c_rdata, s_axi_i2c_rresp, s_axi_i2c_rvalid, s_axi_i2c_rready | Output | — | AXI4-Lite slave write-response, read-address and read-data channel signals |
| tx_data_reg (mem-mapped) | — | 8 | Register: byte to transmit (address or data) |
| rx_data_reg (mem-mapped) | — | 8 | Register: byte received |
| ctrl_reg (mem-mapped) | — | — | Register: start/stop/read-write command bits, clock divider, enable |
| status_reg (mem-mapped) | — | — | Register: busy, ACK/NACK, arbitration-lost, bus-error flags |
| scl_out / scl_oe | Output | 1 | Clock line drive value / output-enable (open-drain to i2c_scl) |
| scl_in | Input | 1 | Sampled clock line value (from i2c_scl) |
| sda_out / sda_oe | Output | 1 | Data line drive value / output-enable (open-drain to i2c_sda) |
| sda_in | Input | 1 | Sampled data line value (from i2c_sda) |
| i2c_irq | Output | 1 | Interrupt request to PIC on transfer completion or error |

---

# 9. PWM Generator

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| s_axi_pwm_awaddr, s_axi_pwm_awvalid, s_axi_pwm_awready, s_axi_pwm_wdata, s_axi_pwm_wstrb, s_axi_pwm_wvalid, s_axi_pwm_wready | Input | — | AXI4-Lite slave write-address and write-data channel signals |
| s_axi_pwm_bresp, s_axi_pwm_bvalid, s_axi_pwm_bready, s_axi_pwm_araddr, s_axi_pwm_arvalid, s_axi_pwm_arready, s_axi_pwm_rdata, s_axi_pwm_rresp, s_axi_pwm_rvalid, s_axi_pwm_rready | Output | — | AXI4-Lite slave write-response, read-address and read-data channel signals |
| period_reg (mem-mapped) | — | 16/32 | Register: waveform period (counter top value) |
| duty_reg (mem-mapped) | — | 16/32 | Register: duty-cycle compare value |
| ctrl_reg (mem-mapped) | — | — | Register: enable, polarity |
| pwm_wave | Output | 1 | Generated PWM waveform (drives pwm_out) |

---

# 10. PIC (Programmable/Platform-Level Interrupt Controller — built into VeeR EL2)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| ext_irq_uart | Input | 1 | Interrupt source from UART (uart_irq) |
| ext_irq_timer | Input | 1 | Interrupt source from Timer (timer_irq) |
| ext_irq_gpio | Input | 1 | Interrupt source from GPIO (gpio_irq) |
| ext_irq_spi | Input | 1 | Interrupt source from SPI Controller (spi_irq) |
| ext_irq_i2c | Input | 1 | Interrupt source from I²C Controller (i2c_irq) |
| meip | Output | 1 | Aggregated machine external-interrupt-pending signal to the core's CSR logic |
| pic_priority_regs (mem-mapped) | — | — | Per-source priority configuration registers (core-programmable) |
| pic_enable_regs (mem-mapped) | — | — | Per-source interrupt-enable registers (core-programmable) |
| pic_claim_reg (mem-mapped) | — | — | Claim/complete register read by the core's ISR to identify and acknowledge the pending source |

---

## Notes

Peripheral register bit-fields (e.g., exact ctrl_reg bit assignments) are left to the detailed micro-architecture/RTL design phase; this list fixes the bus-level and pin-level signal set needed to begin interface and testbench planning.

All AXI4-Lite slave interfaces follow the same signal template (§3b); only the peripheral-specific control/status/serial signals differ block to block.

Inout signals (i2c_scl, i2c_sda, gpio_pins) are shown as split _in/_out/_oe triplets at the RTL block level, consistent with standard tristate-pad modeling practice, and merged into a single bidirectional pin only at the top-level SoC boundary.

AXI path assumption: The peripheral-access path is specified as an AXI4-Lite memory-mapped path. The LSU-facing side uses a 64-bit data channel, while the peripheral-facing side uses a 32-bit data channel; the AXI4-Lite interconnect performs address decoding and any required data-width adaptation. AXI4-Lite is used because the listed peripherals are register-mapped and do not require burst transfers.